Usage: DISK2 {options}

A� leas� on� optio� mus� b� specifie� suc� a� th� driv� type�  �

Yo� ca� mi� an� o� th� options�  DISK2.CM� require� a� leas� 128� �

system.

Options consist of:

m10            Set drive type to Fujitsu 10 Mbyte

m20            Set drive type to Fujitsu 20 Mbyte

m26            Set drive type to Shugart 26 Mbyte

m10m           Set drive type to Memorex 10 Mbyte

drive #        Format selected drive

format         Format headers

data           Write out data fields with E5H

test           Perform a data field test

seek           Perform a seek test

all            Perform format, data test, seek test

skew #         Set skewing of disk to specified number

Thi� optio goe� alon砠 wit蠠 hardwar� �

settings.

Defaults:

I� a� optio� i� no� specifie� the� th� followin� default� �

are used:

M20

skew of 2

sector size 1024

drive 0

no tests or formatting

Examples:

disk�� forma� dat�

disk2 m20 all

disk2 data

disk2 format data drive 1 skew 3 m10    �

#######################################################################################################